ICS 13.030.50

Z 70

GB

中华人民共和国国家标准

National Standard of the People’s Republic of China

GB 16487.13-2005

进口可用作原料的固体废物环境保护控制标准—废汽车压件

Environmental protection control standard for imported solid wastes as raw materials

—Compressed piece of scrap automobile

2005-12-14 Issued	2006-02-01 Implement

Issued by	  State Environmental Protection Administration of China (SEPA)

General Administration of Quality Supervision, Inspection and Quarantine of the People's Republic of China (AQSIQ).

Foreword

This standard is prepared for the purpose of implementing “Law of the People's Republic of China on the Prevention and Control of Environmental Pollution by Solid Waste”, in order to prevent the import of solid wastes that cannot be used as raw materials, standardize the import examination and licensing of solid wastes as raw materials and control the environmental pollution caused by compressed piece of scrap automobile as raw materials.

As one of environmental protection control standards for imported solid wastes, this standard shall be applicable to the import administration of compressed piece of scrap automobile (imported solid wastes as raw materials).

This standard hereby sets forth control requirements for the imported compressed piece of scrap automobile, mainly the requirements for the radioactivity and carried-wastes, and the extent of dismantlement and compression.

According to relevant law, this standard is enforceable.

This standard was proposed by the Department of Science, Technology and Standard, SEPA.

This standard was drafted by Chinese Research Academy of Environmental Sciences.

This standard was approved by SEPA on November 9, 2005.

This standard shall be implemented as of February 1, 2006.

SEPA shall be responsible for interpreting this standard.

Environmental protection control standard for imported solid wastes as raw materials—Compressed piece of scrap automobile

1. Scope

This standard hereby sets forth the environmental protection control requirements for imported compressed piece of scrap automobile, and the control requirements to the extent of dismantlement and compression before the automobile is compressed.

This standard shall be applicable to the import administration of compressed piece of scrap automobile with customs commodity number of 7204.4900.10.

2. Cited normative documents

The clauses in the following documents that were cited by this standard became clauses of this standard. For cited documents with a date, all their subsequent modifications or revisions do not apply to this standard. However, parties having reached an agreement based on cited standards with a date are encouraged to study whether the latest versions of the cited documents with a date are applicable. For cited documents without a date, the latest versions apply to this standard.

GB  5085	Identification standard for hazardous wastes

GB  13015	Pollution control standard for wastes containing polychlorinated biphenyls

SN  0570	Inspection rules for the radioactive contamination of imported wastes as raw materials

National Catalogue of Hazardous Wastes

3 Definitions

This standard shall use the following definitions:

3.1 Compressed piece of scrap automobile shall refer to scrap automobile that has lost its functions, been compressed and can’t restore to its original condition.

3.2 Carried-waste shall refer to substances mixed in imported compressed piece of scrap automobile during the collection, packing and transportation processes (including daily commodities left on the automobile by the driver or riders, exclusive of packing materials for the imported compressed piece of scrap automobile and other substances that need to be used during the transportation process).

4 Standards and requirements for control

4.1 Following parts should be dismantled form compressed piece of scrap automobile, and the total weight of all these parts shall not exceed 0.01% of the weight of the scrap automobile.

（1）Safety airbag;

（2）Storage battery;

（3）Fire extinguisher and sealed pressure container;

（4）Engine oil, gear oil, fuel, fuel gas, brake fluid and cooling fluid;

（5）Refrigerant, catalyst;

（6）Tyre.

4.2 Waste electric motors shall be prohibited from bringing in following carried-wastes (exclusive of wastes listed in Article 4.5).

（1）Radioactive wastes;

（2）Explosive arms and ammunitions such as discarded bomb and shell, etc.;

（3）Wastes containing polychlorinated biphenyls;

（4）Substances identified as hazardous wastes according to GB5085;

（5）Other wastes listed in “National Catalogue of Hazardous Wastes”.

4.3 α and β radioactive contamination limit to the surface of the compressed piece of scrap automobile: For any 300 cm2 of the surface area, the average α specific activity value shall not exceed 0.04Bq/cm2 and that of β shall not exceed 0.4 Bq/cm2.

4.3 The specific activity value of the radionuclide in the compressed piece of scrap automobile shall not exceed the limits specified in List 1.

List 1	Specific activity limits of the radionuclide

| Radionuclide | Specific activity (Bq/g) |
| --- | --- |
| 59Ni | 3x103 |
| 63Ni | 3x103 |
| 54Mn | 0.3 |
| 60Co | 0.3 |
| 65Zn | 0.3 |
| 55F e | 300 |
| 90Sr, | 3 |
| 134Cs | 0.3 |
| 137Cs | 0.3 |
| 235U | 0.3 |
| 238U | 0.3 |
| 239Pu | 0.1 |
| 241Am | 0.3 |
| 152Eu | 0.3 |
| 154Eu | 0.3 |
| 94Nb | 0.3 |
| Unknown β-γ mixture | 0.3 |
| Unknown α mixture | 0.1 |

4.5 Following carried-wastes shall be strictly erstricted and their total weight shall not exceed 0.01% of the weight of compressed piece of scrap automobile.

（1）Asbestos waste or waste containing asbestos;

（2）Sensitization material waste;

（3）Sealed container;

（4）Other hazardous wastes that can’t avoid being mixed into the imported compressed piece of scrap automobile during the production, collection and transportation processes.

4.5 In addition to the wastes listed above, other carried-wastes (such as waste wood, waste paper, thermoplastic, domestic refuse left on the automobile, etc.) shall be restricted and their total weight shall not exceed 1% of the total weight of the imported compressed piece of scrap automobile.

5 Inspections

5.1 Inspections under Article 4.2(3) of this standard shall be performed in accordance with the provisions of GB13015.

5.2 Inspections under Article 4.2(4) and Article 4.2(5) of this standard shall be performed in accordance with the provisions of GB5085.

5.3 Inspections under Article 4.1(1), Article 4.3 and Article 4.4 of this standard shall be performed in accordance with the provisions of SN0570.

5.4 Inspections of other terms under this standard shall be performed in accordance with relevant quality supervision and inspection rules.