ICS 13.030.50

Z 70

GB

中华人民共和国国家标准

National Standard of the People’s Republic of China

GB 16487.11-2005

Replace GB 16487.11-1996

进口可用作原料的固体废物环境保护控制标准—供拆卸的船舶及其他浮动结构体

Environmental protection control standard for imported solid wastes as raw materials

—Vessels and other floating structures for breaking up

2005-12-14 Issued	2006-02-01 Implement

Issued by	  State Environmental Protection Administration of China (SEPA)

General Administration of Quality Supervision, Inspection and Quarantine of the People's Republic of China (AQSIQ).

Foreword

This standard is prepared for the purpose of implementing “Law of the People's Republic of China on the Prevention and Control of Environmental Pollution by Solid Waste”, in order to prevent the import of solid wastes that cannot be used as raw materials, standardize the import examination and licensing of solid wastes as raw materials and control the environmental pollution caused by vessels and other floating structures for breaking up as raw materials.

As one of environmental protection control standards for imported solid wastes, this standard shall be applicable to the import administration of vessels and other floating structures for breaking up (imported solid wastes as raw materials).

This version of standard made many revisions to GB16487.11-1996, such as the name, foreword, scope, cited standards, definition, control requirements, etc. In regard to the control requirements, we mainly revised the control requirements for the radioactivity and carried-wastes, increased the categories of carried-wastes in the scrap ships that are forbidden or prohibited to import, and added control requirements for the asbestos materials in the imported scrap ships.

According to relevant law, this standard is enforceable.

This standard was proposed by the Department of Science, Technology and Standard, SEPA.

This standard was drafted by Chinese Research Academy of Environmental Sciences.

This standard was first issued on July 29, 1996.

This standard was approved by SEPA on November 9, 2005.

This standard shall be implemented as of February 1, 2006.

SEPA shall be responsible for interpreting this standard.

Environmental protection control standard for imported solid wastes as raw materials—Vessels and other floating structures for breaking up

1. Scope

This standard hereby sets forth the environmental protection control requirements for imported vessels and other floating structures for breaking up (hereinafter referred to as “scrap ships”).

This standard shall be applicable to the import administration of scrap ships with customs commodity number of 8908.0000.00.

2. Cited normative documents

The clauses in the following documents that were cited by this standard became clauses of this standard. For cited documents with a date, all their subsequent modifications or revisions do not apply to this standard. However, parties having reached an agreement based on cited standards with a date are encouraged to study whether the latest versions of the cited documents with a date are applicable. For cited documents without a date, the latest versions apply to this standard.

GB  3552	Effluent standard for pollutants from ship.

GB  5085	Identification standard for hazardous wastes

GB  13015	Pollution control standard for wastes containing polychlorinated biphenyls

SN  0570	Inspection rules for the radioactive contamination of imported wastes as raw materials

SN  0578	Inspection and quarantine rules for imported wastes as raw materials—vessels and other floating structures for breaking up.

National Catalogue of Hazardous Wastes

Catalogue of Hazardous Chemicals

Catalogue of Hypertoxic Chemicals

3 Definitions

This standard shall use the following definitions:

3.1 Carried-waste shall refer to domestic refuse made by the crew and the residual waste of the shipped cargo, exclusive of materials that need to be used during the voyage, the cargo and its residue shipped in the rescue ships.

3.2 Light tonnage shall refer to the light displacement of the ship, measuring the weight of the ship itself.

3.3 Hazardous chemical substance shall refer to chemicals listed in the “Catalogue of Hazardous Chemicals” and the “Catalogue of Hypertoxic Chemicals” issued by concerned department of the People’s Republic of China.

4 Standards and requirements for control

4.1 Scrap ships shall be prohibited from bringing in following carried-wastes (exclusive of wastes listed in Article 4.5).

（1）Radioactive wastes;

（2）Explosive arms and ammunitions such as discarded bomb and shell, etc.;

（3）Wastes containing polychlorinated biphenyls

（4）Substances identified as hazardous wastes according to GB5085;

（5）Other wastes listed in “National Catalogue of Hazardous Wastes”.

4.2 The import of scrap oil ships without oil washing process shall be prohibited.

4.3 α and β radioactive contamination limit to the surface of the scrap ships: For any 300 cm2 of the surface area, the average α specific activity value shall not exceed 0.04Bq/cm2 and that of β shall not exceed 0.4 Bq/cm2.

4.4 The specific activity value of the radionuclide in the scrap ships shall not exceed the limits specified in List 1.

List 1	Specific activity limits of the radionuclide

| Radionuclide | Specific activity (Bq/g) |
| --- | --- |
| 59Ni | 3x103 |
| 63Ni | 3x103 |
| 54Mn | 0.3 |
| 60Co | 0.3 |
| 65Zn | 0.3 |
| 55F e | 300 |
| 90Sr, | 3 |
| 134Cs | 0.3 |
| 137Cs | 0.3 |
| 235U | 0.3 |
| 238U | 0.3 |
| 239Pu | 0.1 |
| 241Am | 0.3 |
| 152Eu | 0.3 |
| 154Eu | 0.3 |
| 94Nb | 0.3 |
| Unknown β-γ mixture | 0.3 |
| Unknown α mixture | 0.1 |

4.5 Following carried-wastes shall be strictly restricted and their total weight shall not exceed 0.01%of the weight of scrap ship itself.

（1）Asbestos waste or waste containing asbestos（exclusive of the insulating asbestos material in the ship itself）;

（2）Residual oil and oil sludge in the cabin of scrap ships;

（3）Sensitization material waste;

（4）Sealed containers（exclusive of those of the ship itself）;

（5）Other hazardous wastes that can’t avoid being mixed into the imported scrap ships during the production, collection and transportation processes.

4.6 The weight of insulating asbestos material in the ship itself shall not exceed 0.08% of the weight of the scrap ship itself.

4.7 In addition to the wastes listed above, other carried-wastes in the imported scrap ship which is tugged into China shall be restricted and their total weight shall not exceed 0.05% of the weight of the ship itself.

4.8 For crap ships sailing into China by itself, in addition to the wastes listed above, the formula of the total weight of other carried wastes (hereinafter referred to as Wwaste) is as follows:

Wwaste ≤1.5TN (kg)

(Wwaste ---total weight of the ship wastes (kg); T ---berth time (day); N----crew number (kg/person.day))

4.9 Special cargo ships which had ever shipped cargos listed in Article 4.1 and 4.5 and other hazardous chemicals shall be required to clean. The importers must provide inspection authorities with the names of those cargos and chemical substances that they used to contain.

4.10 The pollutants from the scrap ships shall meet the standards of GB 3552.

5 Inspections

5.1 Inspections under Article 4.1(3) of this standard shall be performed in accordance with the provisions of GB13015.

5.2 Inspections under Article 4.1(4) and Article 4.1(5) of this standard shall be performed in accordance with the provisions of GB5085.

5.3 Inspections under Article 4.1(1), Article 4.3 and Article 4.4 of this standard shall be performed in accordance with the provisions of SN0570.

5.4 Inspections of other terms under this standard shall be performed in accordance with the provisions of SN0578.